# HRNSQ Full Wiring Table
Analog CPG + Digital Modulation + EMI/Filters (All 4 Legs)

Prageeth Madushankha

# Overview

Front & Rear leg pairs — 2 half-center oscillators, 4 neurons total

# Analog CPG — Neuron A (Front Left / Right)

| # | Component | From Pin / Terminal | To Component | To Pin / Terminal | Function |
| --- | --- | --- | --- | --- | --- |
| 1 | TL072 (Neuron A) | Pin 3 (IN+) | FSR divider output | Vout | Sensory bias input |
| 2 | TL072 (Neuron A) | Pin 2 (IN−) | Inhibition resistor (47 kΩ) | Junction | Mutual inhibition |
| 3 | Resistor 47 kΩ | One end | TL072 (Neuron B) | Pin 6 (OUT) | Cross inhibition |
| 4 | Resistor 47 kΩ | Other end | TL072 (Neuron A) | Pin 2 (IN−) | Inhibitory input |
| 5 | TL072 (Neuron A) | Pin 6 (OUT) | Timing capacitor (1 μF) | One terminal | Oscillation timing |
| 6 | Timing capacitor 1 μF | Other terminal | Ground | GND | RC timing |
| 7 | Resistor 22 kΩ | One end | +Vref | Bias | Threshold set |
| 8 | Resistor 22 kΩ | Other end | TL072 (Neuron A) | Pin 3 (IN+) | Threshold set |
| 9 | Potentiometer 50 kΩ | End terminal | TL072 (Neuron A) | Pin 2 (IN−) | Frequency tuning |
| 10 | Potentiometer 50 kΩ | Wiper | TL072 (Neuron A) | Pin 6 (OUT) | Feedback control |
| 11 | TL072 (Neuron A) | Pin 4 (VCC−) | Power supply | −12V / VGND | Negative rail |
| 12 | TL072 (Neuron A) | Pin 7 (VCC+) | Power supply | +12V | Positive rail |
| 13 | Bypass capacitor 0.1 μF | One end | Pin 7 | Supply decoupling | High-frequency decoupling |
| 14 | Bypass capacitor 0.1 μF | Other end | GND | Noise suppression | EMI reduction |
| 15 | Ceramic capacitor 10 nF | Across supply rails | VCC+ / VCC− | Filter | EMI suppression |
| 16 | Ferrite bead | +12V input | TL072 supply | Filter | High-frequency EMI attenuation |

Component count (Neuron A): TL072 ×1, Resistors ×3, Potentiometer ×1, Capacitors ×4, Ferrite bead ×1.

# Analog CPG — Neuron B (Front Left / Right)

| # | Component | From Pin / Terminal | To Component | To Pin / Terminal | Function |
| --- | --- | --- | --- | --- | --- |
| 17 | TL072 (Neuron B) | Pin 3 (IN+) | FSR divider output | Vout | Sensory bias input |
| 18 | TL072 (Neuron B) | Pin 2 (IN−) | Inhibition resistor (47 kΩ) | Junction | Mutual inhibition |
| 19 | Resistor 47 kΩ | One end | TL072 (Neuron A) | Pin 6 (OUT) | Cross inhibition |
| 20 | Resistor 47 kΩ | Other end | TL072 (Neuron B) | Pin 2 (IN−) | Inhibitory input |
| 21 | TL072 (Neuron B) | Pin 6 (OUT) | Timing capacitor (1 μF) | One terminal | Oscillation |
| 22 | Timing capacitor 1 μF | Other terminal | Ground | GND | RC timing |
| 23 | Resistor 22 kΩ | One end | +Vref | Bias | Threshold set |
| 24 | Resistor 22 kΩ | Other end | TL072 (Neuron B) | Pin 3 (IN+) | Threshold set |
| 25 | Potentiometer 50 kΩ | End terminal | TL072 (Neuron B) | Pin 2 (IN−) | Frequency tuning |
| 26 | Potentiometer 50 kΩ | Wiper | TL072 (Neuron B) | Pin 6 (OUT) | Feedback |
| 27 | Diode 1N4148 | Anode | TL072 (Neuron B) | Pin 6 (OUT) | Amplitude limiting |
| 28 | Diode 1N4148 | Cathode | Ground | GND | Protection |
| 29 | TL072 (Neuron B) | Pin 4 (VCC−) | −12V / VGND | Power |  |
| 30 | TL072 (Neuron B) | Pin 7 (VCC+) | +12V | Power |  |
| 31 | Bypass capacitor 0.1 μF | One end | Pin 7 | Supply decoupling | Noise suppression |
| 32 | Bypass capacitor 0.1 μF | Other end | GND | Noise suppression | EMI reduction |
| 33 | Ceramic capacitor 10 nF | Across supply rails | VCC+ / VCC− | Filter | EMI suppression |
| 34 | Ferrite bead | +12V input | TL072 supply | Filter | High-frequency EMI attenuation |

Component count (Neuron B): TL072 ×1, Resistors ×3, Potentiometer ×1, Capacitors ×4, Diodes ×2, Ferrite bead ×1.

# Analog CPG — Neuron C (Rear Left / Right)

| # | Component | From Pin / Terminal | To Component | To Pin / Terminal | Function |
| --- | --- | --- | --- | --- | --- |
| 41 | TL072 (Neuron C) | Pin 3 (IN+) | FSR divider output | Vout | Sensory bias input |
| 42 | TL072 (Neuron C) | Pin 2 (IN−) | Inhibition resistor (47 kΩ) | Junction | Mutual inhibition |
| 43 | Resistor 47 kΩ | One end | TL072 (Neuron D) | Pin 6 (OUT) | Cross inhibition |
| 44 | Resistor 47 kΩ | Other end | TL072 (Neuron C) | Pin 2 (IN−) | Inhibitory input |
| 45 | TL072 (Neuron C) | Pin 6 (OUT) | Timing capacitor (1 μF) | One terminal | Oscillation timing |
| 46 | Timing capacitor 1 μF | Other terminal | Ground | GND | RC timing |
| 47 | Resistor 22 kΩ | One end | +Vref | Bias | Threshold set |
| 48 | Resistor 22 kΩ | Other end | TL072 (Neuron C) | Pin 3 (IN+) | Threshold set |
| 49 | Potentiometer 50 kΩ | End terminal | TL072 (Neuron C) | Pin 2 (IN−) | Frequency tuning |
| 50 | Potentiometer 50 kΩ | Wiper | TL072 (Neuron C) | Pin 6 (OUT) | Feedback control |
| 51 | TL072 (Neuron C) | Pin 4 (VCC−) | Power supply | −12V / VGND | Negative rail |
| 52 | TL072 (Neuron C) | Pin 7 (VCC+) | Power supply | +12V | Positive rail |
| 53 | Bypass capacitor 0.1 μF | One end | Pin 7 | Supply decoupling | High-frequency decoupling |
| 54 | Bypass capacitor 0.1 μF | Other end | GND | Noise suppression | EMI reduction |
| 55 | Ceramic capacitor 10 nF | Across supply rails | VCC+ / VCC− | Filter | EMI suppression |
| 56 | Ferrite bead | +12V input | TL072 supply | Filter | High-frequency EMI attenuation |

Component count (Neuron C): TL072 ×1, Resistors ×3, Potentiometer ×1, Capacitors ×4, Ferrite bead ×1.

# Analog CPG — Neuron D (Rear Left / Right)

| # | Component | From Pin / Terminal | To Component | To Pin / Terminal | Function |
| --- | --- | --- | --- | --- | --- |
| 57 | TL072 (Neuron D) | Pin 3 (IN+) | FSR divider output | Vout | Sensory bias input |
| 58 | TL072 (Neuron D) | Pin 2 (IN−) | Inhibition resistor (47 kΩ) | Junction | Mutual inhibition |
| 59 | Resistor 47 kΩ | One end | TL072 (Neuron C) | Pin 6 (OUT) | Cross inhibition |
| 60 | Resistor 47 kΩ | Other end | TL072 (Neuron D) | Pin 2 (IN−) | Inhibitory input |
| 61 | TL072 (Neuron D) | Pin 6 (OUT) | Timing capacitor (1 μF) | One terminal | Oscillation |
| 62 | Timing capacitor 1 μF | Other terminal | Ground | GND | RC timing |
| 63 | Resistor 22 kΩ | One end | +Vref | Bias | Threshold set |
| 64 | Resistor 22 kΩ | Other end | TL072 (Neuron D) | Pin 3 (IN+) | Threshold set |
| 65 | Potentiometer 50 kΩ | End terminal | TL072 (Neuron D) | Pin 2 (IN−) | Frequency tuning |
| 66 | Potentiometer 50 kΩ | Wiper | TL072 (Neuron D) | Pin 6 (OUT) | Feedback |
| 67 | Diode 1N4148 | Anode | TL072 (Neuron D) | Pin 6 (OUT) | Amplitude limiting |
| 68 | Diode 1N4148 | Cathode | Ground | GND | Protection |
| 69 | TL072 (Neuron D) | Pin 4 (VCC−) | −12V / VGND | Power |  |
| 70 | TL072 (Neuron D) | Pin 7 (VCC+) | +12V | Power |  |
| 71 | Bypass capacitor 0.1 μF | One end | Pin 7 | Supply decoupling | Noise suppression |
| 72 | Bypass capacitor 0.1 μF | Other end | GND | Noise suppression | EMI reduction |
| 73 | Ceramic capacitor 10 nF | Across supply rails | VCC+ / VCC− | Filter | EMI suppression |
| 74 | Ferrite bead | +12V input | TL072 supply | Filter | High-frequency EMI attenuation |

Component count (Neuron D): TL072 ×1, Resistors ×3, Potentiometer ×1, Capacitors ×4, Diodes ×2, Ferrite bead ×1.

# FSR Sensor Interface (All Legs)

| # | Component | From | To | Function |
| --- | --- | --- | --- | --- |
| 75 | FSR (Front Left) | One terminal | +5V | Force sensing |
| 76 | FSR (Front Left) | Other terminal | Divider resistor (22 kΩ) | Voltage divider |
| 77 | FSR (Rear Left) | One terminal | +5V | Force sensing |
| 78 | FSR (Rear Left) | Other terminal | Divider resistor (22 kΩ) | Voltage divider |
| 79 | Resistors 22 kΩ | Other end | GND | Convert force → voltage |
| 80 | Divider output | Junction | TL072 IN+ | Reflex modulation |

Component count: FSR ×4, Resistors ×4.

# Digital Modulation (Cognitive Layer)

| # | Component | From | To | Function |
| --- | --- | --- | --- | --- |
| 81 | Microcontroller | SDA / SCL | DAC | Digital control |
| 82 | DAC | Analog OUT | Resistor 10 kΩ | Signal conditioning |
| 83 | Resistor 10 kΩ | Other end | TL072 IN+ | Modulation input |
| 84 | Capacitor 100 nF | Across DAC output | GND | EMI filtering |
| 85 | Ferrite bead | DAC supply | TL072 supply | High-frequency EMI |

Component count: Microcontroller ×1, DAC ×1, Resistors ×1, Capacitor ×1, Ferrite bead ×1.

# Actuation & Power (All Legs)

| # | Component | From | To | Function |
| --- | --- | --- | --- | --- |
| 86 | Motor driver FL | Control | TL072 OUT (A/B) | Front Left / Right actuation |
| 87 | Motor driver RL | Control | TL072 OUT (C/D) | Rear Left / Right actuation |
| 88 | Motor driver | Output | Servo / Motor | Physical movement |
| 89 | Power supply | +12V | TL072 Pin 7 | Analog rail |
| 90 | Power supply | −12V | TL072 Pin 4 | Analog rail |
| 91 | Bypass capacitor 1 μF | Across rails | +12V / −12V | Power decoupling |
| 92 | Ceramic capacitor 100 nF | Across rails | +12V / −12V | EMI suppression |
| 93 | Ferrite bead | +12V input | Motor driver | EMI suppression |

Component count: Motor driver ×2, Power supply ×1, Capacitors ×2, Ferrite bead ×1.

# Total Component Count (All 4 Legs)

| Component | Quantity |
| --- | --- |
| TL072 | 4 |
| Resistors | 26 |
| Potentiometers | 4 |
| Capacitors | 22 |
| Diodes | 4 |
| Ferrite beads | 4 |
| FSR | 4 |
| DAC | 1 |
| Microcontroller | 1 |
| Motor drivers | 2 |